Ropener Curtain Controller

Bill of Materials

Version 2.0

Everything needed to build one Ropener. Quantities are per unit. Links in the Description column point to the custom PCB and suggested suppliers; equivalent parts may be substituted where noted.

# Body Assembly

| Part | Qty | Description |
| --- | --- | --- |
| VAL3000 PCB | 1 | Custom PCB designed for this project |
| NEMA 17 motor, 48 mm body | 1 | Smaller length NEMA 17 will also work |
| 12V / 2A+ power adapter w/ 5.5×2.1 mm plug | 1 | Technically 5–29 V will work, but 12 V is best due to low speeds |
| Extension cable | 1 | Extends the length of the power adapter |
| M3 × 10 mm screw, self-tapping | 8 |  |
| M3 × 35 mm screw, button head | 3 |  |
| M3 square nut | 1 |  |
| M3 × 16 mm screw, button head | 2 |  |
| M3 × 10 mm screw, flat head | 2 |  |
| Gear | 1 | MK7 |
| Bearing | 2 | 7 mm × 3 mm × 3 mm |

# Curtain Assembly

| Part | Qty | Description |
| --- | --- | --- |
| PTFE tubing, 3 mm ID × 5 mm OD | 1 | 290 mm length required |
| M3 × 10 mm screw, self-tapping | 11 |  |
| M3 square nut | 1 |  |
| M3 × 12 mm screw, flat head | 1 |  |
| Zip tie | 1 | Small, 2–3 mm wide |
| V623ZZ pulley | 1 | 3 × 12 × 4 mm size |
| Rope | 1 | 1.6 mm diameter |

## Mounting Hardware

| Part | Qty | Description |
| --- | --- | --- |
| Command strips | 2 | The easiest way to mount to any wall |
| #17 × 1 1/4" wire nail | 3 | Use to nail to drywall only |

# Assembly Tools Required

| Part | Qty | Description |
| --- | --- | --- |
| Razor | 1 | To cut the length of PTFE tubing |
| Screwdriver, Phillips No. 1 | 1 |  |
| Hex driver, 2 mm | 1 |  |